# @(#)$Copyright:	$

# Copyright (c) 1984, 1985, 1986, 1987, 1988, 1989, 1990

# Sequent Computer Systems, Inc.   All rights reserved.

#

# This software is furnished under a license and may be used

# only in accordance with the terms of that license and with the

# inclusion of the above copyright notice.   This software may not

# be provided or otherwise made available to, or used by, any

# other person.  No title to or ownership of the software is

# hereby transferred.

# @(#)$Header: Remote.doc 1.2 84/12/18 $

This is the protocol used by HOST for remote diagnostics.

HOST is primarily a virtual terminal emulator, and it

watches for packets from the remote.  It then performs

the function(s) requested.  It is essentially a slave

file server and terminal emulator program.

This protocol is modeled after the KERMIT protocol.  It is

modified to be as efficient as possible in the diagnotics

environment while remaining true to the concepts of KERMIT.

The packet format is changed to allow larger packets.  The

actual commands are also changed.  The following definitions

apply.

The packet is:

SOH	BC	SEQ	TYPE	DATA	CHECK

where:

SOH	is a ^A

BC	is the number of remaining bytes, i.e. the

packet length minus 3.

SEQ	is the packet sequence number, modulo 64.

TYPE 	is the packet type.  The following packet

types are supported:

^E	ACK

^U	NAK

S	Script on.

N	Script off.

O	Open a file.

C	Close a file.

R	Read data.

W	Write data.

D	Data

L	Lseek in file.

DATA	is the data.

CHECK	is the checksum of the packet (12 bit quantity).

The following macros are used:

BYTE(x)		((x)+32)

UNBYTE(x)	((x)-32)

CTL(x)		((x)^64)

WORD(x,y)	(UNBYTE(x)+94*UNBYTE(y))

UNWORD(w,p)	{*p++=BYTE((w)%94);*p++=BYTE((w)/94);}

The code assumes that 8 bit paths are provided from UNIX and a 7 bit

path to UNIX for the data field only.  The headers should always be

ok.

A list of control characters to be escaped are maintained.  This list

minimally includes ^S, ^Q, ^P, ^A, ^[, ^B, ^C.

This protocol supports data compression, but only on "D" data packets,

i.e., the compression is really done at a fairly high level, above

the protocol.  Routines which transmit and receive "D" packets

will compress up to 8K of identical characters into 2 or 3 bytes.

The following transactions may occur:

Open file.

Remote sends O packet.  Data in packet is file modes and file

name.  Modes is a 32 bit number transmitted as 8 hex digits,

least significant nibble first.  The file name is an ascii

string.  The transaction is as follows:

R: OPEN, SEQ=0, Data = 8 bytes of how, name

H: ACK, SEQ=0, Data is file number as BYTE(fd)

or

H: ACK, SEQ=0, Data is BYTE(0), message received,

file open failed.

or

H: NAK, SEQ=0, message invalid.

Close file.

Remote sends C packet. Data in packet is the the file number to

close.  The transaction is as follows:

R: CLOSE, SEQ=0, Data is file number as BYTE(fd)

H: ACK, SEQ=0, Data is BYTE(0), file closed successfully.

or

H: ACK, SEQ=0, Data is BYTE(1), , file close failed.

or

H: NAK, SEQ=0, message invalid.

Read file.

Remote sends R packet. Data in packet is the byte count as 32

bit integer sent in hex format, low nibble first.

R: READ, SEQ=0, Data is BYTE(fd) + byte count to read as 8 bytes.

H: ACK, SEQ=0

or

H: NAK, SEQ=0, message invalid.

then:

H: DATA, SEQ=n, Data is from file, compressed and escaped.

R: ACK, SEQ=n

or

R: NAK, SEQ=n

repeat

H: DATA, SEQ=n, no Data

R: ACK, SEQ=n

or

R: NAK, SEQ=n

Write file.

R: WRITE, SEQ=0, Data is BYTE(fd)

H: ACK, SEQ=0

or

H: NAK, SEQ=0, message invalid.

then:

H: DATA, SEQ=n, Data is for file, funny format.

R: ACK, SEQ=n, Data=BYTE(0), write ok

or

R: ACK, SEQ=n, Data=BYTE(1), write failed

or

R: NAK, SEQ=n

repeat

H: DATA, SEQ=n, no Data

R: ACK, SEQ=n

or

R: NAK, SEQ=n

Lseek file.

Remote sends L packet. Data in packet is the file number and

the absolute offset.

R: LSEEK, SEQ=0, Data is BYTE(file), 8(offset), BYTE(whence)

H: ACK, SEQ=0, Data = 8(BYTE)

or

H: NAK, SEQ=0, message invalid

Script on.

Remote sends S packet. Data in packet is the the file name to

open.  The transaction is as follows:

R: S, SEQ=0, Data is file name

H: ACK, SEQ=0, Data is BYTE(0), file opened successfully.

or

H: ACK, SEQ=0, Data is BYTE(1), file open failed.

or

H: NAK, SEQ=0, message invalid.

Script off.

Remote sends N packet.

The transaction is as follows:

R: NOSCRIPT, SEQ=0

H: ACK, SEQ=0

or

H: NAK, SEQ=0, message invalid.